### Qn no.1: Design and implement a secure firewall policy for a university campusnetwork by defining appropriate access-control rules, segmenting networkresources, controlling inter-department communication, and evaluating theeffectiveness of the implemented security policy through connectivity andsecurity testing.

### AIM

To design and implement a secure firewall policy for a university campus network using Cisco Packet Tracer by segmenting network resources, controlling inter-department communication using access-control rules, and testing the effectiveness of the security policy.

### APPARATUS / SOFTWARE REQUIRED

- Cisco Packet Tracer

- Router

- Layer 2 Switch

- PC-PT devices

- Printer-PT devices

- Ethernet cables

### OBJECTIVES

- To design a secure university campus network.

- To divide the network into separate departmental segments.

- To assign appropriate IP addresses to network devices.

- To configure routing between different network segments.

- To control inter-department communication using Access Control Lists (ACLs).

- To allow authorized communication and block unauthorized access.

- To verify the firewall policy through connectivity and security testing.

### PROCEDURE

- Open Cisco Packet Tracer and create the university campus network topology using a router, switch, PCs, and printers.

- Connect the PCs and printers to the switch using Ethernet cables.

- Connect the router to the switch.

- Assign IP addresses to the devices according to their respective network segments.

- Configure the router interfaces and required routing for communication between the networks.

- Configure the required VLAN/network segmentation for different departments.

- Configure Access Control Lists (ACLs) on the router to control communication between departments.

- Allow required communication between authorized networks and block unauthorized communication.

- Test connectivity between the network devices using ping or Packet Tracer simulation tools.

- Verify that allowed traffic is successful and restricted traffic is blocked.

- Observe the packet flow using Simulation Mode and record the results.

OUTPUT

### RESULT

The secure university campus network was successfully designed and simulated using Cisco Packet Tracer. Network segmentation and access-control rules were implemented to control communication between different departments. Connectivity and security testing were performed to verify the effectiveness of the firewall policy.